PROCESS

### Phase 1: Task Injection & The Decentralized Auction (CNP)

- The Injection: Your Bash script acts as the warehouse management system, injecting a new delivery task (pickup and drop-off coordinates) directly into the shared Zenoh P2P topic /swarm/tasks/pool.

- The Battery Gate: Every AMR currently in an IDLE state hears the broadcast. Before they are allowed to bid, they check their battery. If they are below ~25%, they disqualify themselves from the auction and route to a charger.

- The Math & The Bid: Eligible AMRs calculate their driving distance to the pickup point and broadcast a numerical bid score back to the channel within a strict 500ms window.

- Symmetry Breaking: The AMRs compare bids locally. The robot with the best score automatically claims the task and transitions to ASSIGNED. If there is an exact tie, they apply a randomized 10–200ms micro-delay to mathematically force a winner and prevent an auction livelock.

### Phase 2: Cargo Profiling & Intent Broadcast

- Visual Profiling (YOLOv8): Upon arriving at the cargo, the winning robot visually classifies the payload type. It dynamically adjusts its linear interpolation (speed) and P-controller (turning dynamics) for safe handling, and updates its required safety buffer on the Zenoh network.

- The Mapping: The AMR uses its Nav2 stack and the Dynamic A* algorithm to calculate the absolute shortest path to the drop-off point.

- The Swarm Update: Before moving, it broadcasts its planned (x, y, time) trajectory to the /swarm/trajectories topic. The dashboard passively renders this movement without issuing any commands.

### Phase 3: The Drive & Active Edge-AI Guardrails

- Predictive Routing (XGBoost): The swarm constantly analyzes the shared trajectory data. If the XGBoost model predicts that two or more AMRs will collide or congest a specific spot within the next 10 seconds, it triggers proactive bidding and reroutes the robots before the bottleneck physically forms.

- Dynamic Yielding: If AMRs meet at a physical choke point, they cross-reference the Priority Aging formula (wait time + battery) to deterministically decide right-of-way without a central server.

- Obstacle Handling (YOLOv8): The onboard camera actively scans the physical path:

  - Human Worker: Triggers an immediate motor cut-off for human safety.

  - Box / Dead AMR: The robot recalculates a local detour and instantly broadcasts a "no way" alert across the Zenoh network. The entire swarm updates their local costmaps to permanently avoid that impassable aisle.

- The Trust Network (Isolation Forest + YOLO):

  - The Isolation Forest continuously audits Zenoh telemetry. If a glitching robot broadcasts impossible mathematical spikes (like teleporting), it is instantly flagged.

  - For plausible but false coordinates, YOLO physically verifies the location. If the robot isn't visibly there, its data is muted from the swarm's pathfinding math.

### Phase 4: Extreme Fail-Safes & Completion

- Degraded Network: If a robot enters a severe Wi-Fi dead-zone, it falls back to purely local LiDAR/vision navigation and caps its speed at ~40% to proceed safely until it reconnects.

- Ghost Node Timeout: If a robot goes entirely silent, the swarm uses a 2-tier timeout. It waits ~2 seconds for network jitter; if the silence reaches ~6 seconds, the swarm marks the robot as dead and automatically re-auctions its uncompleted task.

- Delivery & Reset: The cargo is dropped off, the AMR transitions back to IDLE, clears its trajectory, and immediately listens to the task pool for the next job.

### The Input (What Feeds the System)

1. The Task Trigger (Digital Input)

- A simple Bash script injects the raw mission data into the Zenoh P2P network (specifically the /swarm/tasks/pool topic).

- This input consists purely of Pickup Coordinates (X, Y) and Drop-off Coordinates (X, Y). There are no routing instructions or commands attached.

2. The Continuous Edge Data (Sensory Input)

Once a robot takes the job, it constantly consumes live physical inputs to survive:

- Visual Data: Live RGB camera feed feeding into the YOLOv8n-nano model to classify payloads, humans, and blocked aisles.

- Spatial Data: LiDAR readings to calculate exact distances to obstacles and feed the Nav2 local costmap.

- Swarm Telemetry: Continuous stream of (x, y, time), battery levels, and hazard alerts from the other robots over the Zenoh channel.

### The Output (What the System Produces)

1. The Physical Output (The Result)

- The successful, physical delivery of the cargo from the pickup point to the drop-off coordinates, executed without a single collision or gridlock.

- The robot safely transitions its state back to IDLE, ready to accept a new input.

2. The Digital Output (Swarm Intelligence & Dashboard)

- Live Visualization: A continuous, real-time stream of (x, y) coordinates and battery statuses broadcasted to the passive web dashboard so human operators can watch the execution.

- Evolved Swarm Knowledge: If the robot encountered a permanent blockage (like a dropped box), the system outputs a "no way" network broadcast. This permanently updates the internal maps of every other robot in the fleet, meaning the swarm finishes the task smarter than when it started.